# Lower Basin Projected Water Use: 2026 - 2028

July 2026: Most Probable 24-Month Study

### 2026

Total projected water use (7.514 maf) – Based on Lake Mead Operating Condition of Level 1 Shortage and water savings contributions under the LB DCP Agreement and IBWC Minute 323.

#### U.S. Contractors: 6.210 maf

##### California: 3.979 maf

- MWD annual diversion of 915 kaf

  - Projected diversion includes the delivery of 28.2 kaf of ICS

- Total California System Conservation of 448.7 kaf

  - CVWD system conservation of 41.5 kaf

  - IID system conservation of 265 kaf

  - Bard system conservation of 9.9 kaf

  - Quechan system conservation of 13 kaf

  - PVID system conservation of 119.3 kaf

- Needles PSCP volume of 145 af

- Binational ICS creation of 18.2 kaf by MWD and IID

##### Arizona: 2.033 maf

- CAP annual diversion of 950 kaf

  - Projected diversion includes a Shortage volume of 320 kaf, and DCP contribution of 192 kaf by CAWCD

- DCP contribution will be made by creating 192 kaf of non-ICS water

- Total non-CAWCD System Conservation of 108.9 kaf

  - FMYN : 13.9 kaf

  - GRIC : 24.8 kaf

  - SCAT : 23.5 kaf

  - Gabrych : 3.2 kaf

  - MVIDD : 14.2 kaf

  - Cathcart : 61 af

  - YMIDD : 23.9 kaf

  - CVIDD : 2.3 kaf

  - Hopi : 3.1 kaf

- Total CAWCD System Conservation of 98.8 kaf

  - ASARCO : 21 kaf

  - Gilbert : 1.6 kaf

  - Glendale : 7 kaf

  - Metro Water District : 5 kaf

  - Peoria : 5 kaf

  - Scottsdale : 4 kaf

  - Tucson : 52.8 kaf

  - Spanish Trail Water Co. : 2.4 kaf

- Bullhead City PSCP volume of 2,477 af

- System water created by the 242 Well Field Expansion Project of 45.0 kaf

- Binational ICS creation of 18.2 kaf by CAWCD

##### Nevada: 0.198 maf

- SNWA annual use of 190 kaf. Projected diversion includes:

  - Shortage volume of 13 kaf

- DCP contribution of 8 kaf through EC ICS conversion

- Total System Conservation of 124 kaf

  - Tributary conservation of 35 kaf

  - Other system conservation of 89 kaf

- Binational ICS creation of 18.2 kaf by SNWA

- Vacated ICS space (to system water) of 58 kaf

  - DCP ICS of 8 kaf converted to system water

  - EC ICS of 50 kaf converted to system water

#### Mexico’s Scheduled Water Delivery: 1.304 maf

- Projected delivery includes:

  - Shortage volume of 50 kaf

  - Recoverable Water Savings Contribution of 30 kaf

- Water Reserve delivery of 21.6 kaf

- Water Reserve creation of 137.7 kaf

### 2027

Total projected water use (8.208 maf) – Based on Lake Mead Operating Condition of Level 2 Shortage and water savings contributions under the LB DCP Agreement and IBWC Minute 323. For modeling purposes, simulated years beyond 2026 assume a continuation of the 2007 Interim Guidelines, the 2019 Colorado River Basin Drought Contingency Plans, and Minute 323, including the Binational Water Scarcity Contingency Plan. Except for certain provisions related to ICS recovery and Upper Basin demand management, operations under these agreements are in effect through 2026. Reclamation initiated the process to develop operations for post-2026 in June 2023, and the modeling assumptions described here are subject to change.

#### U.S. Contractors: 6.862 maf

##### California: 4.412 maf

- MWD annual diversion of 940 kaf

  - Projected diversion includes the delivery of 12.5 kaf of ICS and 17.5 kaf of water stored for CVWD’s share of California’s DCP contribution

- DCP contribution of 250 kaf through EC ICS conversion

- Needles PSCP volume of 145 af

##### Arizona: 2.219 maf

- CAP annual diversion of 940 kaf

  - Projected diversion includes a Shortage volume of 400 kaf, DCP contribution of 240 kaf by CAWCD, and ICS delivery of 70.3 kaf

- DCP contribution will be made by creating 50 kaf of ICS and 190 kaf of non-ICS water

- Total non-CAWCD System Conservation of 7.5 kaf

  - GRIC : 7.5 kaf

- Total CAWCD System Conservation of 3.6 kaf

  - Gilbert : 0.8 kaf

  - Tucson : 2.8 kaf

##### Nevada: 0.230 maf

- SNWA annual use of 220 kaf. Projected diversion includes:

  - Shortage volume of 17 kaf

  - Creation of 53.1 kaf of EC ICS

- DCP contribution of 10 kaf through EC ICS conversion

- Total System Conservation of 35 kaf

  - Tributary conservation of 35 kaf

- Vacated ICS space (to system water) of 10 kaf

  - DCP ICS of 10 kaf converted to system water

#### Mexico’s Scheduled Water Delivery: 1.346 maf

- Projected delivery includes:

  - Shortage volume of 70 kaf

  - Recoverable Water Savings Contribution of 84 kaf

### 2028

Total projected water use (7.961 maf) – Based on Lake Mead Operating Condition of Level 3 Shortage and water savings contributions under the LB DCP Agreement and IBWC Minute 323. For modeling purposes, simulated years beyond 2026 assume a continuation of the 2007 Interim Guidelines, the 2019 Colorado River Basin Drought Contingency Plans, and Minute 323, including the Binational Water Scarcity Contingency Plan. Except for certain provisions related to ICS recovery and Upper Basin demand management, operations under these agreements are in effect through 2026. Reclamation initiated the process to develop operations for post-2026 in June 2023, and the modeling assumptions described here are subject to change.

#### U.S. Contractors: 6.736 maf

##### California: 4.387 maf

- MWD annual diversion of 922 kaf

  - Projected diversion includes the creation of 12.5 kaf of ICS and 24.5 kaf of water stored for CVWD’s share of California’s DCP contribution

- DCP contribution of 350 kaf through EC ICS conversion

- Needles PSCP volume of 145 af

##### Arizona: 2.069 maf

- CAP annual diversion of 789 kaf

  - Projected diversion includes a Shortage volume of 480 kaf, and DCP contribution of 240 kaf by CAWCD

- DCP contribution will be made by creating 25.3 kaf of ICS and 214.7 kaf of non-ICS water

- Total non-CAWCD System Conservation of 7.5 kaf

  - GRIC : 7.5 kaf

- Total CAWCD System Conservation of 3.6 kaf

  - Gilbert : 0.8 kaf

  - Tucson : 2.8 kaf

##### Nevada: 0.280 maf

- SNWA annual use of 270 kaf. Projected diversion includes:

  - Shortage volume of 20 kaf

- DCP contribution of 10 kaf through EC ICS conversion

- Total System Conservation of 35 kaf

  - Tributary conservation of 35 kaf

- Vacated ICS space (to system water) of 37.8 kaf

  - DCP ICS of 10 kaf converted to system water

  - EC ICS of 27.8 kaf converted to system water

#### Mexico’s Scheduled Water Delivery: 1.225 maf

- Projected delivery includes:

  - Shortage volume of 125 kaf

  - Recoverable Water Savings Contribution of 150 kaf

### ICS Totals

- Projected ICS Total Storage at the end of CY 2028: 2.700 maf

- Projected ICS Storage Balances at the end of each calendar year in the study are as follows:

| State (volumes in AF) | 2026 | 2027 | 2028 |
| --- | --- | --- | --- |
| AZ | 600,000 | 574,741 | 600,000 |
| CA | 1,650,000 | 1,637,474 | 1,650,000 |
| NV | 450,000 | 487,785 | 450,000 |
| Total | 2,700,000 | 2,700,000 | 2,700,000 |

### Modeled Lower Basin Conservation Actions

| State | Conservation Activity | 2026 | 2027 | 2028 | Total |
| --- | --- | --- | --- | --- | --- |
| AZ | CAP System Conservation Agreements | 98,800 | 3,567 | 3,567 | 105,934 |
|  | Cathcart Farms System Conservation | 61 | 0 | 0 | 61 |
|  | Cibola Valley IDD System Conservation | 2,329 | 0 | 0 | 2,329 |
|  | Fort McDowell Yavapai Nation System Conservation | 13,933 | 0 | 0 | 13,933 |
|  | Matador System Conservation | 3,240 | 0 | 0 | 3,240 |
|  | GRIC System Conservation | 24,775 | 7,468 | 7,468 | 39,711 |
|  | Hopi Tribe System Conservation | 3,059 | 0 | 0 | 3,059 |
|  | MVIDD System Conservation | 14,184 | 0 | 0 | 14,184 |
|  | San Carlos Apache Tribe System Conservation | 23,451 | 0 | 0 | 23,451 |
|  | YMIDD System Conservation | 23,893 | 0 | 0 | 23,893 |
| CA | Coachella Groundwater System Conservation | 35,000 | 0 | 0 | 35,000 |
|  | Coachella Ag System Conservation | 6,511 | 0 | 0 | 6,511 |
|  | IID System Conservation | 265,000 | 0 | 0 | 265,000 |
|  | MWD ICS Creation | 0 | 0 | 12,527 | 12,527 |
|  | MWD System Conservation | 0 | 0 | 0 | 0 |
|  | PVID-MWD System Conservation | 119,338 | 0 | 0 | 119,338 |
|  | Bard-MWD System Conservation | 9,862 | 0 | 0 | 9,862 |
|  | Quechan Indian Tribe-MWD System Conservation | 13,000 | 0 | 0 | 13,000 |
| NV | SNWA System Conservation Water | 89,000 | 0 | 0 | 89,000 |
|  | SNWA ICS Creation | 0 | 53,095 | 0 | 53,095 |
|  | SNWA Tributary Conservation | 35,000 | 35,000 | 35,000 | 105,000 |
| Other | 242 Wellfield Additional Pumping Agreement | 45,000 | 0 | 0 | 45,000 |
|  | PSCP | 2,622 | 145 | 145 | 2,912 |
|  | Annual Total | 828,058 | 99,275 | 58,706 | 986,039 |
|  | Cumulative Total | 828,058 | 927,333 | 986,039 |  |

| State | 2026 | 2027 | 2028 | Total |
| --- | --- | --- | --- | --- |
| AZ | 255,202 | 11,035 | 11,035 | 277,272 |
| CA | 448,856 | 145 | 12,671 | 461,672 |
| NV | 124,000 | 88,095 | 35,000 | 247,095 |
| Annual Total | 828,058 | 99,275 | 58,706 | 986,039 |
| Cumulative Total | 828,058 | 927,333 | 986,039 |  |

## Notes and Disclaimers

- Modeled Conservation volumes reflect executed agreements and/or current operational projections/assumptions and are subject to change. Additional conservation activities are being considered. After new agreements are finalized and executed, these additional activities will be included in Reclamation’s operational modeling.

- Projected SEIS ROD Reservoir Protection Volume is 3.876 maf from 2023 through 2026.